Department of Electronics and Telecommunication Engineering

VLSI Design

Lab Manual

Vision, Mission and PEOs of E&TC Department

Department Vision:

To be a leading source of Electronics and Telecommunication of globally competent engineers that meets the needs of evolving industry and society.

Department Mission:

- Facilitate learning of cutting-edge technologies in association with industry and contribute towards the latest knowledge generation for better employability and sustainability.

- Promote innovation, research and development enabling higher education, entrepreneurship, and lifelong learning.

- Provide a platform for harnessing leadership qualities imbibed with social and ethical values.

Department Program Educational Objectives:

- PEO1: Graduates will have a strong understanding of the basics of science and engineering as well as analytical skills to solve real world problems.

- PEO2: Graduates will gain technical proficiency in Electronics and Telecommunication field and scale new heights in profession through lifelong learning.

- PEO3: Graduates will embrace professional and ethical manners at all the levels and constantly evolve in multidisciplinary approach leading towards sustainability.

- PEO4: Graduates will utilize engineering acumen with communication skills, leadership qualities and spirit of team work to exhibit qualities for imparting services to society.

List of Experiment

| S. No. | Name of Experiment |
| --- | --- |
| 1. | Implementation of all gates using Gate flow modelling in Verilog. |
| 2. | Implementation of half and full adder using Data flow and Gate level modelling in Verilog. |
| 3. | Implementation of Mux using Data flow, Gate level and Behavioral modelling styles in Verilog. |
| 4. | Implementation of Decoder in Verilog. |
| 5. | Implementation of full adder using instances of half adder in Verilog. |
| 6. | Implementation of Up down counter and Mod 30 counter using Test bench in Verilog. |
| 7. | Implementation of D and T flip flops using Test bench in Verilog. |
| 8. | Implementation of JK and SR flip flops using Test bench in Verilog. |
| 9. | Implementation of shift registers using Test bench in Verilog. |
| 10. | Implementation of 4 bit Ripple Carry Counter using instances of T & D Flip Flops. |
| 11. | Implementation of Moore machine using Test bench in Verilog. |
| 12. | Implementation of Mealy machine using Test bench in Verilog. |
| 13. | Implementation of ALU using Test bench in Verilog. |

Experiment No. 1

Aim: Implementation of all gates using Gate flow modelling in Verilog.

Software Tool: Vivado Design Suite

Truth Table:

| A | B | AND | NAND | OR | NOR | XOR | XNOR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 |
| 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 1 | 0 | 0 | 1 |

Procedure:

- Open the Xilinx Vivado Design Suite

- Go to file and click new project

- Enter the project name and click next

- Select the family name of the FPGA Device, parameter setting and HDL is verilog click next and click finish.

- Click new source.

- Select Verilog module and type file name and click next.

- Assign input and output port and click next.

- Finally, the report is shown click finish.

- Type the program save and check syntax error.

- To see the output waveform select ISim simulator

- Give values to the input variables using force clock or force constant and then click run

- In wave window, click run icon and you can see corresponding output.

- For synthesis of the design, open XST synthesis tool and run the design for synthesis.

- Open RTL schematic and Technology schematic and understand implemented design on FPGA

- Open synthesis result to know resource utilization of the design.

Code:

module allgates(

input a, b,

output nd, nnd, r, nr, xr, nxr, nt

);

and (nd, a, b);

nand (nnd, a, b);

or (r, a, b);

nor (nr, a, b);

xor (xr, a, b);

xnor (nxr, a, b);

not (nt, a);

endmodule

Result

- Simulate the design using the simulation tool to verify its functionality.

- Synthesis the design using synthesis tool to find out the resource utilization.

- Draw RTL and Technology schematic.

Conclusion:

Questions for Reflection:

- What do you mean by HDL?

- What are different modelling styles in Verilog?

- What is the syntax of gate level modelling?

- What is the structure of Verilog coding?

- What do you mean by concurrent and sequential programming?

- Can you write only y output for all gates? Comment on the error if output is denoted by y for all gates.

- Is there any effect on resource utilization if we change FPGA device for Synthesis?

Experiment No. 2

Aim: Implementation of half and full adder using gate level and data flow modeling in Verilog

Apparatus:  Xilinx ISE Design Suite

Theory: Truth table of half adder and full adder

Full Adder

Procedure:

- Open the Xilinx Vivado Design Suite

- Go to file and click new project

- Enter the project name and click next

- Select the family name of the FPGA Device, parameter setting and HDL is verilog click next and click finish.

- Click new source.

- Select Verilog module and type file name and click next.

- Assign input and output port and click next.

- Finally, the report is shown click finish.

- Type the program save and check syntax error.

- To see the output waveform select ISim simulator

- Give values to the input variables using force clock or force constant and then click run

- In wave window, click run icon and you can see corresponding output.

- For synthesis of the design, open XST synthesis tool and run the design for synthesis.

- Open RTL schematic and Technology schematic and understand implemented design on FPGA

- Open synthesis result to know resource utilization of the design.

Code:

## HALF ADDER

| Module ha_glm (input a, input b, output sum, output c_out ); xor x1(sum,a,b); and n1(c_out,a,b); endmodule | module ha_dflm(input a, input b, output s, output c); assign s=a^b; assign c=a&b; endmodule |
| --- | --- |

Gate level modeling	             Dataflow modeling

## FULL ADDER

| module fa_glm ( input x, input y, input cin, output A, output cout ); wire p,r,s; xor(p,x,y); xor (A,p,cin); and(r,p,cin); and(s,x,y); or(cout,r,s); endmodule | module fsd_dlm ( input x, input y, input cin, output A, output cout); assign A=(x^y)^cin; assign cout =((x^y)&cin)¦(x&y); endmodule |
| --- | --- |

Gate level modeling	                Dataflow modeling

Result

- Simulate the design using the simulation tool to verify its functionality.

- Synthesis the design using synthesis tool to find out the resource utilization.

- Draw RTL and Technology schematic.

Conclusion:

Questions for Reflection:

- What is the difference between data flow and gate level modelling?

- What do you mean reg, wire and net?

- What is the requirement for gate level modeling and data flow modeling?

- What do you mean by assign statement in data flow modeling?

- What are different data flow operators?

- Can you assign FPGA pins to wire signal?

- Is there any difference between simulation result of data flow and gate level modeling of Half and full adder?

- Is there any difference between synthesis result of data flow and gate level modeling of Half and full adder?

Experiment 3

Aim: Implementation of full adder using 2 instances of half adder.

Apparatus:  Xilinx Vivado Design Suite

Theory: Truth table of full adder

| X | Y | Z | SUM | CARRY |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

- Open the Xilinx Vivado Design Suite

- Go to file and click new project

- Enter the project name and click next

- Select the family name of the FPGA Device, parameter setting and HDL is verilog click next and click finish.

- Click new source.

- Select Verilog module and type file name and click next.

- Assign input and output port and click next.

- Finally, the report is shown click finish.

- Type the program save and check syntax error.

- To see the output waveform select ISim simulator

- Give values to the input variables using force clock or force constant and then click run

- In wave window, click run icon and you can see corresponding output.

- For synthesis of the design, open XST synthesis tool and run the design for synthesis.

- Open RTL schematic and Technology schematic and understand implemented design on FPGA

- Open synthesis result to know resource utilization of the design.

Code:

Using Positional Mapping

module FA_using_HA(

input x, y, z,

output s, c

);

wire s1, c1, c2;

HA A1(x, y, s1, c1);

HA A2(s1, z, s, c2);

or (c, c1, c2);

endmodule

module HA(

input a, b,

output s, c

);

xor (s, a, b);

and (c, a , b);

endmodule

Using name mapping

module FA_using_HA(

input x, y, z,

output s, c

);

wire s1, c1, c2;

HA_dot A1(.a(x), .b(y), .s(s1), .c(c1));

HA_dot A2(.a(s1), .b(z), .s(s), .c(c2));

or (c, c1, c2);

endmodule

module HA_dot(

input a, b,

output s, c

);

xor (s, a, b);

and (c, a , b);

endmodule

Result:

- Simulate the design using the simulation tool to verify its functionality.

- Synthesis the design using synthesis tool to find out the resource utilization.

- Draw RTL and Technology schematic of the design.

Conclusion:

Questions for Reflection:

- What do you understand by instance mapping?

- What are different types of instance mapping?

- What are the rules for instance mapping?

- How the mapping of the variables will take place in positional mapping?

- How the mapping of the variables will take place in name mapping?

- What is the use of dot operator in name mapping?

- Is there any difference in synthesis design if we use instance mapping (Compare your result with earlier synthesis and schematic result.

Experiment 4

Aim: Implementation of 8:1 mux using:

a) Dataflow modeling

b) Gate level modeling

c) Behavioral Modelling

Apparatus:  Xilinx Vivado Design Suite

Theory: Truth table of 8:1 Mux

| S2 | S1 | S0 | Y |
| --- | --- | --- | --- |
| 0 | 0 | 0 | I0 |
| 0 | 0 | 1 | I1 |
| 0 | 1 | 0 | I2 |
| 0 | 1 | 1 | I3 |
| 1 | 0 | 0 | I4 |
| 1 | 0 | 1 | I5 |
| 1 | 1 | 0 | I6 |
| 1 | 1 | 1 | I7 |

Procedure:

- Open the Xilinx Vivado Design Suite

- Go to file and click new project

- Enter the project name and click next

- Select the family name of the FPGA Device, parameter setting and HDL is verilog click next and click finish.

- Click new source.

- Select Verilog module and type file name and click next.

- Assign input and output port and click next.

- Finally, the report is shown click finish.

- Type the program save and check syntax error.

- To see the output waveform select ISim simulator

- Give values to the input variables using force clock or force constant and then click run

- In wave window, click run icon and you can see corresponding output.

- For synthesis of the design, open XST synthesis tool and run the design for synthesis.

- Open RTL schematic and Technology schematic and understand implemented design on FPGA

- Open synthesis result to know resource utilization of the design.

Code:

Gate Level Modelling

module mux81_gate (

input [7:0] I,

input S0,S1,S2,

output Y

);

wire [11:1] T;

not(T[1], S0);

not(T[2], S1);

not(T[3], S2);

and(T[4], I[0], T[1], T[2], T[3]), (T[5], I[1], S0, T[2], T[3]);

and(T[6], I[2], T[1], S1, T[3]), (T[7], I[3], S0, S1, T[3]);

and(T[8], I[4], T[1], T[2], S2), (T[9], I[5], S0, T[2], S2);

and(T[10], I[6], T[1], S1, S2), (T[11], I[7], S0, S1, S2);

or(Y, T[4], T[5], T[6], T[7], T[8], T[9], T[10], T[11]);

endmodule

Dataflow Modelling

module mux81_dataflow(

input i0,i1,i2,i3,i4,i5,i6,i7,

input s0,s1,s2,

output out

);

assign out = s2?(s1?(s0?i7:i6):(s0?i5:i4)):(s1?(s0?i3:i2):(s0?i1:i0));

endmodule

module mux8x1(

input [7:0] i,

input [2:0] s,

output Y

);

wire [7:0] y;

assign y[0] = ~s[2] & ~s[1] & ~s[0] & i[0];

assign y[1] = ~s[2] & ~s[1] &  s[0] & i[1];

assign y[2] = ~s[2] &  s[1] & ~s[0] & i[2];

assign y[3] = ~s[2] &  s[1] &  s[0] & i[3];

assign y[4] =  s[2] & ~s[1] & ~s[0] & i[4];

assign y[5] =  s[2] & ~s[1] &  s[0] & i[5];

assign y[6] =  s[2] &  s[1] & ~s[0] & i[6];

assign y[7] =  s[2] &  s[1] &  s[0] & i[7];

assign Y = y[0] | y[1] | y[2] | y[3] | y[4] | y[5] | y[6] | y[7];

endmodule

Behavioral Modelling:

module mux8x1(

input [7:0] i,

input [2:0] s,

output y

);

reg y;

always@(s or i)

case(s)

3'b000: y = i[0];

3'b001: y = i[1];

3'b010: y = i[2];

3'b011: y = i[3];

3'b100: y = i[4];

3'b101: y = i[5];

3'b110: y = i[6];

3'b111: y = i[7];

endcase

endmodule

Result:

- Simulate the design using the simulation tool to verify its functionality.

- Synthesis the design using synthesis tool to find out the resource utilization.

- Draw RTL and Technology schematics of the design.

Conclusion:

Questions for Reflection:

- What are the constraints for data flow modelling style?

- What are the constraints for gate level modeling style?

- What is conditional data operator?

- What do you mean by [7:0]i?

- How execution of assign statement will take place?

- Whether all assign statements execute sequentially or concurrently?

- Why behavioral modelling is called highest abstraction level?

- What do you mean by always @

- Is there any difference in RTL and technology schematics of the various modelling styles of Verilog?

- Is there any difference in synthesis design for different codes used in the program (Compare your result with 3 codes written in the program)

Experiment 5

Aim: Implementation of 3:8 decoder using Behavioral Modelling in Verilog.

Software:  Xilinx Vivado Design Suite

Theory: Truth table of 8:1 Mux

| IN2 | IN1 | IN0 | OUT7 | OUT6 | OUT5 | OUT4 | OUT3 | OUT2 | OUT1 | OUT0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Procedure:

- Open the Xilinx Vivado Design Suite

- Go to file and click new project

- Enter the project name and click next

- Select the family name of the FPGA Device, parameter setting and HDL is verilog click next and click finish.

- Click new source.

- Select Verilog module and type file name and click next.

- Assign input and output port and click next.

- Finally, the report is shown click finish.

- Type the program save and check syntax error.

- To see the output waveform select ISim simulator

- Give values to the input variables using force clock or force constant and then click run

- In wave window, click run icon and you can see corresponding output.

- For synthesis of the design, open XST synthesis tool and run the design for synthesis.

- Open RTL schematic and Technology schematic and understand implemented design on FPGA

- Open synthesis result to know resource utilization of the design.

Code:

module decoder3to8_ (

input [2:0] in,

output reg [7:0] out

);

always @(in)

case (in)

3'b000 : out = 8'b00000001;

3'b001 : out = 8'b00000010;

3'b010 : out = 8'b00000100;

3'b011 : out = 8'b00001000;

3'b100 : out = 8'b00010000;

3'b101 : out = 8'b00100000;

3'b110 : out = 8'b01000000;

3'b111 : out = 8'b10000000;

default : out = 8'b00000000;

endcase

endmodule

Result:

- Simulate the design using the simulation tool to verify its functionality.

- Synthesis the design using synthesis tool to find out the resource utilization.

- Draw RTL and Technology schematics of the design.

Conclusion:

Questions for Reflection:

- Why reg is required for output variable in Behavioral modelling?

- What is the meaning of always@(*)?

- How case statement is executed?

- Change the code using if- else condition.

Experiment 6

Aim: Implementation of 4 bit up- down counter and Mod 30 using Behavioral Modelling in Verilog.

Software:  Xilinx Vivado Design Suite

Circuit Diagram:

Procedure:

- Open the Xilinx Vivado Design Suite

- Go to file and click new project

- Enter the project name and click next

- Select the family name of the FPGA Device, parameter setting and HDL is verilog click next and click finish.

- Click new source.

- Select Verilog module and type file name and click next.

- Assign input and output port and click next.

- Finally, the report is shown click finish.

- Type the program save and check syntax error.

- To see the output waveform select ISim simulator

- Give values to the input variables using force clock or force constant and then click run

- In wave window, click run icon and you can see corresponding output.

- For synthesis of the design, open XST synthesis tool and run the design for synthesis.

- Open RTL schematic and Technology schematic and understand implemented design on FPGA

- Open synthesis result to know resource utilization of the design.

Code:

module updowncounter (clk, reset, UpOrDown, Count);

input clk, reset, M;

output [3 : 0] Count;

reg[3 : 0] Count =0;

always @(posedge clk)

begin

if(reset == 1)

Count <= 0;

else

if(M == 1)

if(Count == 15)

Count <=0;

else

Count <= Count+1;

else

if(Count==0)

Count <= 15;

else

Count <= Count -1;

end

endmodule

Test Bench

module updowncounter

reg clk;

reg reset;

reg UpOrDown;

// Outputs

wire [3:0] Count;

// Instantiate the Unit Under Test (UUT)

updowncounter_sid_113 uut (

.clk(clk),

.reset(reset),

.UpOrDown(UpOrDown),

.Count(Count)

);

initial clk = 0;

always #5 clk = ~clk;

initial begin

// Apply Inputs

reset = 1;

UpOrDown = 0;

#300;

reset = 0;

UpOrDown = 1;

#100;

UpOrDown = 0;

end

initial

$monitor("reset=%0d UpOrDown=%0d Count=%0d simtime=%g clk-%b",reset ,UpOrDown ,Count ,$time , clk);

endmodule

Mod 30 Counter code:

module m30 (

input clk, reset,

output reg [4:0] Q

);

always @ (posedge clk)

if (reset)

begin Q <= 5'b00000; end

else

begin

if (Q == 5'b11110)

begin Q <= 5'b0; end

else

begin Q <= Q+1; end

end

endmodule

Test Bench:

module testmod30;

// Inputs

reg clk;

reg reset;

// Outputs

wire [4:0] Q;

// Instantiate the Unit Under Test (UUT)

m30  uut (

.clk(clk),

.reset(reset),

.Q(Q)

);

initial clk = 0;

always #5 clk = ~clk;

initial begin

reset = 1;

#50;

reset = 0;

end

initial

$monitor("reset=%0d simtime=%g clk-%b",reset ,$time , clk);

endmodule

Result:

- Simulate the design using the simulation tool to verify its functionality.

- Synthesis the design using synthesis tool to find out the resource utilization.

- Draw RTL and Technology schematics of the design.

Conclusion:

Questions for Reflection:

- What is the need of test bench?

- How input and output is defined in the test bench?

- What is the use of dot operator in test bench?

- What are the rules to define module and variables in the test bench?

- What is the function of initial block in the test bench?

- Is there any restriction for number of initial blocks in the test bench?

- How the execution of various initial blocks will take place in the Verilog modelling?

- How the execution of statements written inside the initial block will take place?

- What is the difference between blocking and non-blocking statements?

- What is the use of $monitor in the test bench?

Experiment 7

Aim: Implementation of D and T flip flops in Verilog.

Software:  Xilinx Vivado Design Suite

Truth Table:

Circuit Diagram:

Procedure:

- Open the Xilinx Vivado Design Suite

- Go to file and click new project

- Enter the project name and click next

- Select the family name of the FPGA Device, parameter setting and HDL is verilog click next and click finish.

- Click new source.

- Select Verilog module and type file name and click next.

- Assign input and output port and click next.

- Finally, the report is shown click finish.

- Type the program save and check syntax error.

- To see the output waveform select ISim simulator

- Give values to the input variables using force clock or force constant and then click run

- In wave window, click run icon and you can see corresponding output.

- For synthesis of the design, open XST synthesis tool and run the design for synthesis.

- Open RTL schematic and Technology schematic and understand implemented design on FPGA

- Open synthesis result to know resource utilization of the design.

Code:

D flip flop

module d_ff (

input clk, d, set, reset,

output reg q

);

always @ (posedge clk or posedge set or posedge reset)

begin

if (set)

q <= 1′b1;

else if (reset)

q <= 1′b0;

else

q <= d;

end

endmodule

Test bench:

D flip flop

Code

module dff_test_siddharth_113;

// Inputs

reg clk;

reg d;

reg set;

reg reset;

// Outputs

wire q;

// Instantiate the Unit Under Test (UUT)

d_ff uut (

.clk(clk),

.d(d),

.set(set),

.reset(reset),

.q(q)

);

initial begin

clk=0;

forever #10 clk = ~clk;

end

initial begin

reset=1; d <= 0;

#100; set=1; d <= 1;

#100; d <= 0;

#100; d <= 1;

$monitor("clk = %b, d = %b,reset = %b,set = %b, q = %b",clk, d, reset, set, q);

end

endmodule

T flip flop

module t_ff(

input clk, t, set, reset,

output reg q

);

always @ (posedge clk or posedge set or posedge reset)

begin

if (set)

q <= 1'b1;

Siddharth Sameer 113

4

else if (reset)

q <= 1'b0;

else

q <= ~t;

end

endmodule

Test Bench:

T flip flop

Code

module tff_test;

// Inputs

reg clk;

reg t;

reg set;

reg reset;

// Outputs

wire q;

// Instantiate the Unit Under Test (UUT)

t_ff  uut (

.clk(clk),

.t(t),

.set(set),

.reset(reset),

.q(q)

);

initial begin

clk=0;

forever #10 clk = ~clk;

end

initial begin

reset=1; t <= 0;

#100; set=1; t <= 1;

#100; t <= 0;

#100; t <= 1;

$monitor("clk = %b, t = %b,reset = %b,set = %b, q = %b",clk, t, reset, set, q);

end

endmodule

Result:

- Simulate the design using the simulation tool to verify its functionality.

- Synthesis the design using synthesis tool to find out the resource utilization.

- Draw RTL and Technology schematics of the design.

Conclusion:

Questions for Reflection:

- What is the need of test bench?

- How input and output is defined in the test bench?

- What is the use of dot operator in test bench?

- What do you mean by #100 in the test bench code?

- What is the alternative for forever?

- What are the rules to define module and variables in the test bench?

- What is the function of initial block in the test bench?

- Is there any restriction for number of initial blocks in the test bench?

- How the execution of various initial blocks will take place in the Verilog modelling?

- How the execution of statements written inside the initial block will take place?

- What is the difference between blocking and non-blocking statements?

- Is test bench code synthesizable?

Experiment 8

Aim: Implementation of SR and JK flip flops in Verilog.

Software:  Xilinx Vivado Design Suite

Truth Table:

Circuit Diagram:

Procedure:

- Open the Xilinx Vivado Design Suite

- Go to file and click new project

- Enter the project name and click next

- Select the family name of the FPGA Device, parameter setting and HDL is verilog click next and click finish.

- Click new source.

- Select Verilog module and type file name and click next.

- Assign input and output port and click next.

- Finally, the report is shown click finish.

- Type the program save and check syntax error.

- To see the output waveform select ISim simulator

- Give values to the input variables using force clock or force constant and then click run

- In wave window, click run icon and you can see corresponding output.

- For synthesis of the design, open XST synthesis tool and run the design for synthesis.

- Open RTL schematic and Technology schematic and understand implemented design on FPGA

- Open synthesis result to know resource utilization of the design.

JK flip flop Code:

module jkff (input j,k,clk,

output reg q );

always @ (posedge clk)

case ({j,k})

2'b00 : q <= q;

2'b01 : q <= 0;

2'b10 : q <= 1;

2'b11 : q <= ~q;

endcase

endmodule

module jkff_test;

// Inputs

reg j;

reg k;

reg clk;

// Outputs

wire q;

always #5 clk = ~clk;

// Instantiate the Unit Under Test (UUT)

Jkff uut (

.j(j),

.k(k),

.clk(clk),

.q(q)

);

initial begin

j <= 0;

k <= 0;

#5 j <= 0;

k <= 1;

#20 j <= 1;

k <= 0;

#20 j <= 1;

k <= 1;

#20 $finish;

end

initial

$monitor ("j=%0d k=%0d q=%0d", j, k, q);

Endmodule

SR FF Code:

module srff  (

s,r,clk, q

);

input s,r,clk;

output reg q;

always@(posedge clk)

begin

if(s == 1)

begin

q <= 1;

end

else if(r == 1)

begin

q <= 0;

end

else if(s == 0 & r == 0)

begin

q <= q;

end

end

endmodule

module srff_test;

// Inputs

reg s;

reg r;

reg clk;

// Outputs

wire q;

// Instantiate the Unit Under Test (UUT)

srff_ uut (

.s(s),

.r(r),

.clk(clk),

.q(q)

);

initial

$monitor("s = %b, r = %b, q = %b",s, r, q);

initial begin

clk=0;

forever #10 clk = ~clk;

end

initial begin

s= 1; r= 0;

#100; s= 0; r= 1;

#100; s= 0; r= 0;

#100; s= 1; r=1;

end

endmodule

Result:

- Simulate the design using the simulation tool to verify its functionality.

- Synthesis the design using synthesis tool to find out the resource utilization.

- Draw RTL and Technology schematics of the design.

Conclusion:

Questions for Reflection:

- What is the need of test bench?

- How input and output is defined in the test bench?

- What is the use of dot operator in test bench?

- What do you mean by #100 in the test bench code?

- What is the alternative for forever?

- What are the rules to define module and variables in the test bench?

- What is the function of initial block in the test bench?

- Is there any restriction for number of initial blocks in the test bench?

- How the execution of various initial blocks will take place in the Verilog modelling?

- How the execution of statements written inside the initial block will take place?

- What is the difference between blocking and non-blocking statements?

- Is test bench code synthesizable?

Experiment 9

Aim: Implementation of shift registers in Verilog.

Software:  Xilinx Vivado Design Suite

Procedure:

- Open the Xilinx Vivado Design Suite

- Go to file and click new project

- Enter the project name and click next

- Select the family name of the FPGA Device, parameter setting and HDL is verilog click next and click finish.

- Click new source.

- Select Verilog module and type file name and click next.

- Assign input and output port and click next.

- Finally, the report is shown click finish.

- Type the program save and check syntax error.

- To see the output waveform select ISim simulator

- Give values to the input variables using force clock or force constant and then click run

- In wave window, click run icon and you can see corresponding output.

- For synthesis of the design, open XST synthesis tool and run the design for synthesis.

- Open RTL schematic and Technology schematic and understand implemented design on FPGA

- Open synthesis result to know resource utilization of the design.

Codes:

a) SISO

module siso(clk, SI, SO);

input clk,SI;

output SO;

reg [7:0] tmp;

always @(posedge clk)

begin

tmp = tmp << 1;

tmp[0] = SI;

end

assign SO = tmp[7];

endmodule

b) SIPO

module sipo(clk, SI, PO);

input clk,SI;

output [7:0] PO;

reg [7:0] tmp;

always @(posedge clk)

begin

tmp = {tmp[6:0], SI};

end

assign PO = tmp;

endmodule

c) PIPO

module pipo(din,clk,rst,dout);

input [3:0] din;

input clk,rst;

output [3:0] dout;

wire [3:0] din;

wire clk,rst;

reg [3:0] dout;

always @(posedge clk or negedge rst)

begin

if(!rst)

begin

dout <= 4'b0;

end

else

begin

dout <= din;

end

end

endmodule

Testbench for PIPO

module testbench_pipo;

// Inputs

reg [3:0] din;

reg clk;

reg rst;

// Outputs

wire [3:0] dout;

// Instantiate the Unit Under Test (UUT)

pipo uut (

.din(din),

.clk(clk),

.rst(rst),

.dout(dout)

);

initial begin

// Initialize Inputs

clk = 1'b0;

rst = 1'b0;

end

always

begin

#20 clk = ~clk;

end

initial

begin

#25 rst = 1'b1;

#10 din = 4'b1010;

#90 din = 4'b0110;

#500 $finish;

end

endmodule

d) PISO

module piso(din,clk,rst,dout);

input [3:0] din;

input clk,rst;

output [3:0] dout;

wire [3:0] din;

wire clk,rst;

reg [3:0] dout;

always @(posedge clk or negedge rst)

begin

if(!rst)

begin

dout <= 4'b0;

end

else

begin

dout <= din;

end

end

endmodule

Testbench for PISO

module testbench_piso;

// Inputs

reg clk;

reg [3:0] PI;

reg load;

// Outputs

wire SO;

// Instantiate the Unit Under Test (UUT)

piso uut (

.clk(clk),

.PI(PI),

.load(load),

.SO(SO)

);

initial begin

// Initialize Inputs

clk = 0;

end

always

#10 clk = ~clk;

initial

begin

#25 load = 1;

#10 PI = 4'b1010;

#20 load = 0;

#100 load = 1;

#20 PI = 4'b1110;

#20 load = 0;

#300 $finish;

end

endmodule

Result:

- Simulate the design using the simulation tool to verify its functionality.

- Synthesis the design using synthesis tool to find out the resource utilization.

- Draw RTL and Technology schematics of the design.

Conclusion:

Questions for Reflection:

- What is the need of test bench?

- How input and output is defined in the test bench?

- What is the use of dot operator in test bench?

- What do you mean by #100 in the test bench code?

- What is the alternative for forever?

- What are the rules to define module and variables in the test bench?

- What is the function of initial block in the test bench?

- Is there any restriction for number of initial blocks in the test bench?

- How the execution of various initial blocks will take place in the Verilog modelling?

- How the execution of statements written inside the initial block will take place?

- What is the difference between blocking and non-blocking statements?

- Is test bench code synthesizable?

Experiment 10

Aim: Implementation of 4 bit Ripple counter using instances of T and D Flip flops

Software:  Xilinx Vivado Design Suite

Truth Table:

Circuit Diagram:

Procedure:

- Open the Xilinx Vivado Design Suite

- Go to file and click new project

- Enter the project name and click next

- Select the family name of the FPGA Device, parameter setting and HDL is verilog click next and click finish.

- Click new source.

- Select Verilog module and type file name and click next.

- Assign input and output port and click next.

- Finally, the report is shown click finish.

- Type the program save and check syntax error.

- To see the output waveform select ISim simulator

- Give values to the input variables using force clock or force constant and then click run

- In wave window, click run icon and you can see corresponding output.

- For synthesis of the design, open XST synthesis tool and run the design for synthesis.

- Open RTL schematic and Technology schematic and understand implemented design on FPGA

- Open synthesis result to know resource utilization of the design.

Code:

module ripple_carry_counter (q, clk, reset);

output [3:0] q;

input clk, reset;

T_FF tff0(q[0],clk, reset);

T_FF tff1(q[1],q[0], reset);

T_FF tff2(q[2],q[1], reset);

T_FF tff3(q[3],q[2], reset);

endmodule

module T_FF(q, clk, reset);

output q;

input clk, reset;

wire d;

D_FF dff0(q, d, clk, reset); // Instantiate D_FF. Call it dff0.

not n1(d, q);

endmodule

module D_FF(q, d, clk,reset);

output q;

input d, clk, reset;

reg q;

always @(posedge reset or negedge clk)

if (reset)

q <= 1'b0;

else

q <= d;

endmodule

Testbench:

module ripple_carry_counter_tb;

reg clk, reset;

wire [3:0] q;

ripple_carry_counter ripple_carry_counter_inst(

.q(q), .clk(clk), .reset(reset)

);

initial clk = 0;

always #10 clk = ~clk;

initial

begin

reset = 1'b1;

#20 reset = 1'b0;

end

always @(posedge clk)

begin

$display("q = %b", q);

end

endmodule

Result:

- Simulate the design using the simulation tool to verify its functionality.

- Synthesis the design using synthesis tool to find out the resource utilization.

- Draw RTL and Technology schematics of the design.

Conclusion:

Questions for Reflection:

- Explain hierarchy of the design?

- What do you mean by top down and bottom-up approach of the design?

- Which approach have you used in your design?

- Have you created 3 different codes in the design and how these codes are linked with each other?

- What are the rules to be followed for positional mapping?

- How can you create instance using name mapping?

Experiment 11

Aim: Implementation of Mealy FSM in Verilog.

Software:  Xilinx Vivado Design Suite

State Diagram and State Table

Procedure:

- Open the Xilinx Vivado Design Suite

- Go to file and click new project

- Enter the project name and click next

- Select the family name of the FPGA Device, parameter setting and HDL is verilog click next and click finish.

- Click new source.

- Select Verilog module and type file name and click next.

- Assign input and output port and click next.

- Finally, the report is shown click finish.

- Type the program save and check syntax error.

- To see the output waveform select ISim simulator

- Give values to the input variables using force clock or force constant and then click run

- In wave window, click run icon and you can see corresponding output.

- For synthesis of the design, open XST synthesis tool and run the design for synthesis.

- Open RTL schematic and Technology schematic and understand implemented design on FPGA

- Open synthesis result to know resource utilization of the design.

Code:

module mealy1101(clk,reset,in,out);

input clk,reset,in;

output out;

reg out;

reg[2:0]pst,nst;

parameter

s0=3'b000,

s1=3'b001,

s2=3'b010,

s3=3'b011;

always @(posedge clk or posedge reset)

begin

if(reset==1)

pst<=s0;

else

pst<=nst;

end

always @(pst or in)

begin

case(pst)

s0:

if(in==1)

begin

nst<=s1;

out<=0;

end

else

nst<=pst;

s1:

if(in==1)

begin

nst<=s2;

out<=0;

end

else

begin

nst<=s0;

out<=0;

end

s2:

if(in==0)

begin

nst<=s3;

out<=0;

end

else

begin

nst<=s2;

out<=0;

end

s3:

if(in==1)

begin

nst<=s1;

out<=1;

end

else

begin

nst<=s0;

out<=0;

end

default

nst<=s0;

endcase

end

endmodule

Test bench:

module testbench_mealy1101;

// Inputs

reg clk;

reg reset;

reg in;

// Outputs

wire out;

// Instantiate the Unit Under Test (UUT)

mealy1101 uut (

.clk(clk),

.reset(reset),

.in(in),

.out(out)

);

initial begin

// Initialize Inputs

clk = 0;

forever #5 clk=~clk;

end

initial begin

reset = 1;

in = 0;

#10 reset=0;

#20 in=0;

#20 in=1;

#10 in=1;

#10 in=0;

#10 in=1;

#10 in=0;

#20 in=0;

#20 in=1;

#10 in=1;

#10 in=0;

#10 in=1;

#100$finish;

end

endmodule

Result:

- Simulate the design using the simulation tool to verify its functionality.

- Synthesis the design using synthesis tool to find out the resource utilization.

- Draw RTL and Technology schematics of the design.

Conclusion:

Questions for Reflection:

- What is the difference between Mealy and Moore FSM?

- Which FSM is more stable and why?

- Draw FSM diagram for sequence detector 1100.

- What do you mean by overlapping and non overlapping FSM?

Experiment 12

Aim: Implementation of Moore FSM in Verilog.

Software:  Xilinx Vivado Design Suite

State Diagram and State Table

Procedure:

- Open the Xilinx Vivado Design Suite

- Go to file and click new project

- Enter the project name and click next

- Select the family name of the FPGA Device, parameter setting and HDL is verilog click next and click finish.

- Click new source.

- Select Verilog module and type file name and click next.

- Assign input and output port and click next.

- Finally, the report is shown click finish.

- Type the program save and check syntax error.

- To see the output waveform select ISim simulator

- Give values to the input variables using force clock or force constant and then click run

- In wave window, click run icon and you can see corresponding output.

- For synthesis of the design, open XST synthesis tool and run the design for synthesis.

- Open RTL schematic and Technology schematic and understand implemented design on FPGA

- Open synthesis result to know resource utilization of the design.

Code:

module moore1010(clk,reset,in,out);

input clk,reset,in;

output out;

reg out;

reg[2:0]pst,nst;

parameter

s0=3'b000,

s1=3'b001,

s2=3'b010,

s3=3'b011,

s4=3'b100;

always @(posedge clk or posedge reset)

begin

if(reset==1)

pst<=s0;

else

pst<=nst;

end

always @(pst or in)

begin

case(pst)

s0:

if(in==1)

nst<=s1;

else

nst<=s0;

s1:

if(in==0)

nst<=s2;

else

nst<=s1;

s2:

if(in==1)

nst<=s3;

else

nst<=s0;

s3:

if(in==0)

nst<=s4;

else

nst<=s1;

s4:

if(in==1)

nst<=s3;

else

nst<=s0;

default

nst<=s0;

endcase

end

always@(pst)// moore logic

begin

if(pst==s4)

out<=1;

else

out<=0;

end

endmodule

Test bench:

module tt_moore_1010;

// Inputs

reg clk;

reg reset;

reg in;

// Outputs

wire out;

// Instantiate the Unit Under Test (UUT)

moore1010 uut (

.clk(clk),

.reset(reset),

.in(in),

.out(out)

);

initial

begin

reset=0;clk=0;in=0;

#5 in=0;

#5 in=0;

#5 in=1;

#10 in=0;

#10 in=1;

#10 in=0;

#10 in=1;

#10 in=0;

#10 in=0;

#5 in=0;

#5 in=1;

end

always

#5 clk=~clk;

endmodule

Result:

- Simulate the design using the simulation tool to verify its functionality.

- Synthesis the design using synthesis tool to find out the resource utilization.

- Draw RTL and Technology schematics of the design.

Conclusion:

Questions for Reflection:

- What is the difference between Mealy and Moore FSM?

- Which FSM is more stable and why?

- Draw FSM diagram for sequence detector 1100.

- What do you mean by overlapping and non overlapping FSM?

Experiment 13

Aim: Implementation of 8 bit ALU in Verilog.

Software:  Xilinx Vivado Design Suite

Truth Table:

Circuit Diagram:

Procedure:

- Open the Xilinx Vivado Design Suite

- Go to file and click new project

- Enter the project name and click next

- Select the family name of the FPGA Device, parameter setting and HDL is verilog click next and click finish.

- Click new source.

- Select Verilog module and type file name and click next.

- Assign input and output port and click next.

- Finally, the report is shown click finish.

- Type the program save and check syntax error.

- To see the output waveform select ISim simulator

- Give values to the input variables using force clock or force constant and then click run

- In wave window, click run icon and you can see corresponding output.

- For synthesis of the design, open XST synthesis tool and run the design for synthesis.

- Open RTL schematic and Technology schematic and understand implemented design on FPGA

- Open synthesis result to know resource utilization of the design.

Code:

Code of the program:

`timescale 1ns / 1ps

module  alu (a, b, s, yout, cf);

input [7:0] a;

wire [7:0] a;

input [7:0] b;

wire [7:0] b;

input [2:0] s;

wire [2:0] s;

output [7:0] yout;

reg [7:0] yout;

output cf;

reg cf;

reg [8:0] temp;

always @ ( a,b,s )

begin

if ( s == 3'b000)

begin

temp = {0,a} + {0,b};

yout = temp [7:0];

cf = temp [8];

end

else if ( s == 3'b001)

begin

temp = {0,a}

- {0,b};

yout = temp [7:0];

cf = temp [8];

end

else if ( s == 3'b010)

begin

temp = {0,a} +5'b00001;

yout = temp [7:0];

cf = temp [8];

end

else if ( s == 3'b011)

begin

temp = {0,a}

- 5'b00001;

yout = temp [7:0];

cf = temp [8];

end

else if ( s == 3'b100)

begin

temp = a | b;

yout = temp [7:0];

cf = temp [8];

end

else if ( s == 3'b101)

begin

temp = a & b;

yout = temp [7:0];

cf = temp [8];

end

else if ( s == 3'b110)

begin

temp = a ^ b;

yout = temp [7:0];

cf = temp [8];

end

else if (s == 3'b100)

begin

temp = ~a;

yout = temp [7:0];

cf = temp [8];

end

end

endmodule

Testbench

`timescale 1ns / 1ps

Module test;

// Inputs

reg [7:0] a;

reg [7:0] b;

reg [2:0] s;

// Outputs

wire [7:0] yout;

wire cf;

// Instantiate the Unit Under Test (UUT)

Alu  uut (

.a(a),

.b(b),

.s(s),

.yout(yout),

.cf(cf)

);

initial begin

$display("Inputs and Outputs are:");

$monitor("time= %t a=%b b=%b s=%b yout=%b cf=%b",$time,a,b,s,yout,cf);

end

initial begin

// Initialize Inputs

a = 0;

b = 0;

s = 0;

// Wait 100 ns for global reset to finish

#100;

a = 1;

b = 2;

s = 0;

#6

a = 1;

b = 2;

s = 2;

#6

a = 2;

b = 3;

s = 3;

#6

a = 6;

b = 3;

s = 4;

#6

a = 1;

b = 2;

s = 5;

#6

a = 2;

b = 4;

s = 6;

#6

a = 1;

b = 2;

s = 7;

#6

a = 5;

b = 2;

s = 1;

#6 $finish;

end

endmodule

Result:

- Simulate the design using the simulation tool to verify its functionality.

- Synthesis the design using synthesis tool to find out the resource utilization.

- Draw RTL and Technology schematics of the design.

Conclusion:

Questions for Reflection:

- What is the difference between $monitor and $ display?

- What is the difference between $finish and $ stop?

- What is default size of int variable?